How to modify the working frequency of the wireless AP module

- Connect the serial port: Use the USB to UART tool, connect the TX, RX, and GND cables to the red box in the figure below.

- Use the SecurECRT tool to open the serial port. Set the session options with a baud rate of 115200. Set to new line mode:

After printing, enter the at+command\r to see if there is a response, such as at+mode\r

3. View channel list:

AT+CHAN_ LIST\r

Return the result, such as:

+CHAN_ LIST: 9080,9160,9240

Ok

4. Configure channel list:

AT+CHAN_ LIST=Center frequency of Channel 1, Center frequency of Channel 2, Center frequency of Channel 3\ r

For example, if configured with only two channels:

AT+CHAN_ LIST=9160,9240\r

To configure as a channel:

AT+CHAN_ LIST=9240\r

5,Modify the scope of work:

AT+CHAN_ LIST=9080,9160,9240

If you want to modify the center frequency to 9080, the setting method is:

AT+CHAN_ LIST=9080\r

6. Modify bandwidth:

To modify the bandwidth to 8M, enter: at+bss_bw=8\r